# Supplementary materials

# Figure S1 : Rock-Eval material and methods detailed

Rock-Eval® thermal analysis was initially developed for petroleum exploration, but applied to the study of SOM over the last 20 years in order to assess the total Corg and Cinorg concentration (Disnar et al., 2003). For this study, finely ground and dried samples (approximately 65 mg) were analyzed using a Rock-Eval® 6 instrument at the IFPEN laboratory (Rueil-Malmaison), following the procedure described in Behar et al. (2001).  During the first step of pyrolysis, samples were heated from 200 to 650°C with a gradient of 25°C per minute in an inert atmosphere (N2). Hydrocarbon compounds (HC), CO and CO2 released by thermal cracking of pyrolysable OM were monitored by the Infrared Detector (IR), respectively. During the oxidation second step, the residues from pyrolysis were heated from 200 to 850°C with a gradient of 25°C per minute under artificial air (N2:O2 ratio 80:20). The CO and CO2 released by combustion of organic compounds were monitored by the IR detector. These measurements provided five thermograms for each sample, showing the evolution of compound release during the pyrolytic and oxidative heating. Using these thermograms, thermolabile (TOC) and thermoresistant (MINC) contents can be calculated by integrating the quantities of hydrocarbon compounds, CO and CO2 evolved between the determined temperature limits (Lafargue et al., 1998, Behar et al., 2001). In addition, the thermograms obtained from the oxidation step can be used to determine the presence or absence of carbonates (peak after 650°C at the CO2 oxidative thermogram). In the absence of carbonates, Corg is calculated by summing the TOC and MINC parameters. The results obtained for Ctot with Rock-Eval® (Corg+Cinorg) are very close to those obtained with catalytic combustion, which confirms the viability of Rock-Eval® for the study of soil carbon.. S2 (mg HC g-1) and S3 (mg CO2 g-1) parameters provide information on the composition of the SOM. S2 corresponds to the quantity of hydrocarbons related to the less mature aliphatic compounds and S3 to the CO2 released during pyrolysis related to the more mature and stable aromatic part of the SOM (Behar et al., 2001). The index R, between 0 and 1, quantifies the part of the S2 thermogram integrated after 400°C and measures the contribution of the most thermally stable SOM corresponding to the most persistent SOM fraction (Sebag et al., 2016). For these parameters, the analytical error was below 10%.

# Figure S2 : Ctot catalytic combustion vs Ctot Rock-Eval ®

# Figure S3: Calibrated date Shaitan III profil 21-2. S60 : 60 cm depth, S125 : 125 cm depth, S143 : 143 cm depth, S170 : 170 cm depth

# Figure S4 : Sample/Upper Crust diagrams Ket. A - topsoils, B - subsoils

# Figure S5 : Sample/Upper Crust diagrams Shaitan III.